## 🩹 Step-by-Step Fix

### Step 1: Boot into Rescue or Multi-User Mode

- Reboot your system.

- When the GRUB menu appears, press e to edit the boot entry.

- Find the line that starts with:

linux ...

- At the end of that line, add:

systemd.unit=multi-user.target

- Press Ctrl + X to boot.

You’ll boot into a non-graphical terminal (CLI mode).

### Step 2: Check Driver Modules

Run:

lsmod | grep nvidia
lsmod | grep nouveau

- If both appear → conflict.

- If only nouveau appears → NVIDIA driver failed to load.

- If only nvidia appears but still broken → possibly GDM/X11 config issue.

## 🩹 Step-by-Step Fix for RHEL 9 NVIDIA Driver Not Loading

### Step 1: Verify the Kernel You’re Running

Run:

uname -r

Then list the NVIDIA DKMS modules built for it:

sudo dkms status

If you see something like:

nvidia/550.90.07, 5.14.0-362.el9.x86_64: built

and your current kernel is different (e.g. 5.14.0-383.el9), that’s the issue — the module isn’t built for the current kernel.

👉 Fix: rebuild the module

sudo dkms autoinstall

NOTE: When we ran for the first we got an error, we tried again and it worked and installed.

### Step 2: Check Secure Boot

Run:

mokutil --sb-state

If you see:

SecureBoot enabled

→ The kernel refuses to load the unsigned NVIDIA driver.

👉 Fix it by:

- Rebooting → entering BIOS → disabling Secure Boot.
 Then reboot and check again:

mokutil --sb-state

It should say “disabled”.

### Step 3: Rebuild initramfs

Sometimes even after fixing the module, RHEL still loads an empty initramfs.

Run:

sudo dracut --force

Then reboot:

sudo reboot

After reboot, check:

lsmod | grep nvidia

and

nvidia-smi